import tkinter as tk

from tkinter import ttk, messagebox

import struct

import math

APP_TITLE = "NanoSense Architecture Simulator"

# ----------------------------- UTILITY FUNCTIONS -----------------------------

def to_twos(value, bits):

return value & ((1 << bits) - 1)

def bin_fixed(value, bits):

return format(to_twos(value, bits), f"0{bits}b")

def signed_from_bits(value, bits):

if value & (1 << (bits - 1)):

return value - (1 << bits)

return value

def float32_bits(value):

packed = struct.pack(">f", float(value))

integer = struct.unpack(">I", packed)[0]

return format(integer, "032b"), integer

def ieee_fields(value):

bits, _ = float32_bits(value)

sign = bits[0]

exponent_bits = bits[1:9]

mantissa = bits[9:]

exponent = int(exponent_bits, 2)

return bits, sign, exponent_bits, exponent, mantissa

def clear_tree(tree):

for item in tree.get_children():

tree.delete(item)

def parse_int(text):

return int(text.strip(), 0)

def parse_accesses(text):

raw = text.replace(",", " ").split()

return [int(x, 0) for x in raw]

# ----------------------------- BOOTH ALGORITHM -------------------------------

def booth_multiply_trace(multiplicand, multiplier, bits):

mask = (1 << bits) - 1

A = 0

Q = to_twos(multiplier, bits)

M = to_twos(multiplicand, bits)

q_minus_1 = 0

trace = []

for step in range(1, bits + 1):

q0 = Q & 1

operation = "No operation"

if q0 == 0 and q_minus_1 == 1:

A = (A + M) & mask

operation = "A = A + M"

elif q0 == 1 and q_minus_1 == 0:

A = (A - M) & mask

operation = "A = A - M"

before_shift_A = A

before_shift_Q = Q

combined = ((A & mask) << (bits + 1)) | ((Q & mask) << 1) | q_minus_1

sign_bit = (combined >> (2 * bits)) & 1

combined >>= 1

if sign_bit:

combined |= 1 << (2 * bits)

A = (combined >> (bits + 1)) & mask

Q = (combined >> 1) & mask

q_minus_1 = combined & 1

trace.append({

"step": step,

"q0q1": f"{q0}{q_minus_1}",

"operation": operation,

"a_before": bin_fixed(before_shift_A, bits),

"q_before": bin_fixed(before_shift_Q, bits),

"a": bin_fixed(A, bits),

"q": bin_fixed(Q, bits),

"qm1": q_minus_1

})

product_bits = (A << bits) | Q

product = signed_from_bits(product_bits, bits * 2)

return trace, product, format(product_bits, f"0{bits*2}b")

# ---------------------------- RESTORING DIVISION -----------------------------

def restoring_division_trace(dividend, divisor, bits):

if divisor == 0:

raise ZeroDivisionError("Division by zero is not allowed.")

sign = -1 if (dividend < 0) ^ (divisor < 0) else 1

a = abs(dividend)

q = abs(divisor)

if a >= (1 << bits) or q >= (1 << bits):

raise ValueError(f"Use values that fit within {bits}-bit magnitude.")

A = 0

Q = a

M = q

mask = (1 << bits) - 1

trace = []

for step in range(1, bits + 1):

combined = ((A & mask) << bits) | (Q & mask)

combined = (combined << 1) & ((1 << (bits * 2)) - 1)

A = (combined >> bits) & mask

Q = combined & mask

a_before_sub = A

signed_A = A - M

if signed_A < 0:

A = a_before_sub

Q = Q & ~1

action = "A - M < 0 → Restore A, Q0 = 0"

else:

A = signed_A

Q = Q | 1

action = "A - M ≥ 0 → Keep A, Q0 = 1"

trace.append({

"step": step,

"a": format(A, f"0{bits}b"),

"q": format(Q, f"0{bits}b"),

"action": action

})

quotient = sign * Q

remainder = abs(dividend) % abs(divisor)

if dividend < 0:

remainder = -remainder

return trace, quotient, remainder

# ------------------------------ PIPELINE MODEL -------------------------------

def parse_instruction(line):

line = line.strip().upper().replace(",", " ")

if not line:

return None

parts = line.split()

op = parts[0]

args = parts[1:]

reads, writes = set(), None

if op in {"ADD", "SUB", "MUL", "AND", "OR"} and len(args) >= 3:

writes = args[0]

reads = {args[1], args[2]}

elif op == "LOAD" and len(args) >= 2:

writes = args[0]

elif op == "STORE" and len(args) >= 2:

reads = {args[0]}

elif op in {"BEQ", "BNE"} and len(args) >= 2:

reads = {args[0], args[1]}

elif op in {"J", "JMP"}:

pass

return {"text": line, "op": op, "reads": reads, "writes": writes}

def simulate_pipeline(lines, forwarding):

instructions = [parse_instruction(x) for x in lines if x.strip()]

instructions = [x for x in instructions if x]

if not instructions:

raise ValueError("Enter at least one instruction.")

start_cycles = []

hazards = []

stalls_before = 0

stalls_after = 0

for i, inst in enumerate(instructions):

stall = 0

for j in range(max(0, i - 2), i):

prev = instructions[j]

if prev["writes"] and prev["writes"] in inst["reads"]:

distance = i - j

if prev["op"] == "LOAD" and distance == 1:

raw_stall = 1

hazard_type = "RAW Load-Use"

elif distance == 1:

raw_stall = 1

hazard_type = "RAW Data"

else:

raw_stall = 0

hazard_type = "RAW Data"

if raw_stall:

hazards.append((i + 1, j + 1, hazard_type,

f"{prev['text']} → {inst['text']}"))

if forwarding:

if prev["op"] == "LOAD" and distance == 1:

stall = max(stall, 1)

else:

stall = max(stall, raw_stall + (1 if prev["op"] == "LOAD" else 0))

if inst["op"] in {"BEQ", "BNE", "J", "JMP"}:

hazards.append((i + 1, "-", "Control Hazard", inst["text"]))

stall += 1

start = (start_cycles[-1] + 1 if start_cycles else 1) + stall

start_cycles.append(start)

if forwarding:

stalls_after += stall

else:

stalls_before += stall

cycles = max(start_cycles) + 4

rows = []

for i, inst in enumerate(instructions):

row = {"instruction": inst["text"], "start": start_cycles[i]}

rows.append(row)

return rows, hazards, cycles, (stalls_after if forwarding else stalls_before)

def build_pipeline_matrix(rows):

total = max(r["start"] + 4 for r in rows)

matrix = []

stages = ["IF", "ID", "EX", "MEM", "WB"]

for r in rows:

row = []

for cycle in range(1, total + 1):

offset = cycle - r["start"]

row.append(stages[offset] if 0 <= offset < 5 else "")

matrix.append(row)

return matrix, total

# ------------------------------- CACHE MODEL ---------------------------------

def direct_mapped(accesses, lines, block_size):

cache = [None] * lines

results = []

for address in accesses:

block = address // block_size

index = block % lines

tag = block // lines

hit = cache[index] == tag

if not hit:

cache[index] = tag

results.append((address, block, index, tag, "HIT" if hit else "MISS"))

return results

def two_way_lru(accesses, sets, block_size):

cache = [[] for _ in range(sets)]

results = []

for address in accesses:

block = address // block_size

index = block % sets

tag = block // sets

ways = cache[index]

if tag in ways:

ways.remove(tag)

ways.append(tag)

result = "HIT"

else:

if len(ways) >= 2:

ways.pop(0)

ways.append(tag)

result = "MISS"

results.append((address, block, index, tag, result))

return results

def cache_metrics(results, hit_time, miss_penalty):

hits = sum(1 for r in results if r[-1] == "HIT")

misses = len(results) - hits

hit_ratio = hits / len(results) if results else 0

amat = hit_time + (1 - hit_ratio) * miss_penalty

return hits, misses, hit_ratio, amat

# -------------------------------- MAIN APP -----------------------------------

class NanoSenseApp(tk.Tk):

def __init__(self):

super().__init__()

self.title(APP_TITLE)

self.geometry("1250x760")

self.minsize(1050, 650)

self.style = ttk.Style(self)

try:

self.style.theme_use("clam")

except Exception:

pass

self.style.configure("Title.TLabel", font=("Arial", 20, "bold"))

self.style.configure("Header.TLabel", font=("Arial", 14, "bold"))

self.style.configure("Result.TLabel", font=("Arial", 11, "bold"))

self.style.configure("Nav.TButton", font=("Arial", 11), padding=10)

self.style.configure("TButton", padding=6)

self.sidebar = ttk.Frame(self, width=220)

self.sidebar.pack(side="left", fill="y", padx=8, pady=8)

self.content = ttk.Frame(self)

self.content.pack(side="right", expand=True, fill="both", padx=(0, 8), pady=8)

ttk.Label(self.sidebar, text="NanoSense", style="Title.TLabel").pack(pady=(12, 0))

ttk.Label(self.sidebar, text="Architecture Simulator").pack(pady=(0, 20))

buttons = [

("🏠 Dashboard", self.show_dashboard),

("🧮 Booth Multiplier", self.show_booth),

("➗ Division", self.show_division),

("🔢 IEEE-754", self.show_ieee),

("⚙ Pipeline", self.show_pipeline),

("💾 Cache Memory", self.show_cache),

("🔌 I/O Strategy", self.show_io),

]

for text, command in buttons:

ttk.Button(self.sidebar, text=text, command=command,

style="Nav.TButton").pack(fill="x", padx=8, pady=4)

ttk.Separator(self.sidebar).pack(fill="x", padx=8, pady=15)

ttk.Label(self.sidebar, text="CSA1215 / Computer Architecture",

wraplength=190, justify="center").pack(padx=8)

self.current = None

self.show_dashboard()

def new_page(self):

if self.current is not None:

self.current.destroy()

self.current = ttk.Frame(self.content)

self.current.pack(expand=True, fill="both")

return self.current

def make_tree(self, parent, columns, headings, height=12):

frame = ttk.Frame(parent)

frame.pack(expand=True, fill="both", pady=8)

tree = ttk.Treeview(frame, columns=columns, show="headings", height=height)

for col, heading in zip(columns, headings):

tree.heading(col, text=heading)

tree.column(col, anchor="center", width=120)

y = ttk.Scrollbar(frame, orient="vertical", command=tree.yview)

x = ttk.Scrollbar(frame, orient="horizontal", command=tree.xview)

tree.configure(yscrollcommand=y.set, xscrollcommand=x.set)

tree.grid(row=0, column=0, sticky="nsew")

y.grid(row=0, column=1, sticky="ns")

x.grid(row=1, column=0, sticky="ew")

frame.rowconfigure(0, weight=1)

frame.columnconfigure(0, weight=1)

return tree

# ---------------------------- DASHBOARD ---------------------------------

def show_dashboard(self):

page = self.new_page()

ttk.Label(page, text="NanoSense Architecture Simulator",

style="Title.TLabel").pack(pady=(35, 8))

ttk.Label(page,

text="Low-Power IoT Edge Sensor Node | Interactive Computer Architecture Simulation",

font=("Arial", 11)).pack(pady=5)

info = ttk.LabelFrame(page, text="Assignment Modules", padding=20)

info.pack(padx=60, pady=25, fill="both", expand=True)

modules = [

("1. Booth's Multiplication", "8-bit / 16-bit signed multiplication with step-by-step trace."),

("2. Restoring Division", "Binary division with quotient and remainder trace."),

("3. IEEE-754 Arithmetic", "Single-precision addition/subtraction with normalization and special cases."),

("4. 5-Stage Pipeline", "IF-ID-EX-MEM-WB hazard detection and forwarding comparison."),

("5. Cache Simulator", "Direct-mapped vs 2-way set-associative hit/miss and AMAT comparison."),

("6. I/O Strategy", "Interrupt-driven versus DMA-based analysis for sensor streaming."),

]

for title, desc in modules:

row = ttk.Frame(info)

row.pack(fill="x", pady=7)

ttk.Label(row, text=title, style="Header.TLabel").pack(anchor="w")

ttk.Label(row, text=desc, wraplength=800).pack(anchor="w")

# ------------------------------ BOOTH -----------------------------------

def show_booth(self):

page = self.new_page()

ttk.Label(page, text="Booth's Signed Multiplication Simulator",

style="Title.TLabel").pack(pady=12)

controls = ttk.LabelFrame(page, text="Input", padding=12)

controls.pack(fill="x", padx=10)

ttk.Label(controls, text="Multiplicand").grid(row=0, column=0, padx=5, pady=5)

m_entry = ttk.Entry(controls, width=15)

m_entry.insert(0, "23")

m_entry.grid(row=0, column=1, padx=5)

ttk.Label(controls, text="Multiplier").grid(row=0, column=2, padx=5)

q_entry = ttk.Entry(controls, width=15)

q_entry.insert(0, "-7")

q_entry.grid(row=0, column=3, padx=5)

ttk.Label(controls, text="Bit Size").grid(row=0, column=4, padx=5)

bits_var = tk.StringVar(value="8")

ttk.Combobox(controls, textvariable=bits_var, values=["8", "16"],

state="readonly", width=8).grid(row=0, column=5, padx=5)

result_var = tk.StringVar(value="Enter values and run the simulation.")

ttk.Label(page, textvariable=result_var, style="Result.TLabel").pack(pady=8)

tree = self.make_tree(page,

("step", "operation", "a_before", "q_before", "a", "q", "qm1"),

("Step", "Operation", "A Before", "Q Before", "A After", "Q After", "Q-1"),

height=14)

def run():

try:

m = parse_int(m_entry.get())

q = parse_int(q_entry.get())

bits = int(bits_var.get())

min_v, max_v = -(1 << (bits - 1)), (1 << (bits - 1)) - 1

if not (min_v <= m <= max_v and min_v <= q <= max_v):

raise ValueError(f"For {bits}-bit signed values, use {min_v} to {max_v}.")

trace, product, product_bits = booth_multiply_trace(m, q, bits)

clear_tree(tree)

for row in trace:

tree.insert("", "end", values=(

row["step"], row["operation"], row["a_before"],

row["q_before"], row["a"], row["q"], row["qm1"]

))

result_var.set(

f"Final Product = {product} | {bits*2}-bit Binary = {product_bits}"

)

except Exception as e:

messagebox.showerror("Input Error", str(e))

ttk.Button(controls, text="Run Booth Simulation", command=run).grid(

row=0, column=6, padx=12)

# ----------------------------- DIVISION ----------------------------------

def show_division(self):

page = self.new_page()

ttk.Label(page, text="Restoring Division Simulator",

style="Title.TLabel").pack(pady=12)

controls = ttk.LabelFrame(page, text="Input", padding=12)

controls.pack(fill="x", padx=10)

ttk.Label(controls, text="Dividend").grid(row=0, column=0, padx=5, pady=5)

dividend_entry = ttk.Entry(controls, width=15)

dividend_entry.insert(0, "100")

dividend_entry.grid(row=0, column=1)

ttk.Label(controls, text="Divisor").grid(row=0, column=2, padx=5)

divisor_entry = ttk.Entry(controls, width=15)

divisor_entry.insert(0, "7")

divisor_entry.grid(row=0, column=3)

ttk.Label(controls, text="Bit Size").grid(row=0, column=4, padx=5)

bits_var = tk.StringVar(value="8")

ttk.Combobox(controls, textvariable=bits_var, values=["8", "16"],

state="readonly", width=8).grid(row=0, column=5)

result_var = tk.StringVar(value="Restoring division uses magnitude internally and restores sign in the final quotient.")

ttk.Label(page, textvariable=result_var, style="Result.TLabel",

wraplength=1000).pack(pady=8)

tree = self.make_tree(page, ("step", "a", "q", "action"),

("Step", "A", "Q", "Action"), height=14)

def run():

try:

dividend = parse_int(dividend_entry.get())

divisor = parse_int(divisor_entry.get())

bits = int(bits_var.get())

trace, quotient, remainder = restoring_division_trace(dividend, divisor, bits)

clear_tree(tree)

for row in trace:

tree.insert("", "end", values=(

row["step"], row["a"], row["q"], row["action"]

))

result_var.set(

f"Quotient = {quotient} | Remainder = {remainder}"

)

except Exception as e:

messagebox.showerror("Division Error", str(e))

ttk.Button(controls, text="Run Division", command=run).grid(row=0, column=6, padx=12)

# ------------------------------ IEEE ------------------------------------

def show_ieee(self):

page = self.new_page()

ttk.Label(page, text="IEEE-754 Single Precision Floating-Point Simulator",

style="Title.TLabel").pack(pady=12)

controls = ttk.LabelFrame(page, text="Input", padding=12)

controls.pack(fill="x", padx=10)

ttk.Label(controls, text="Sensor Reading A").grid(row=0, column=0, padx=5)

a_entry = ttk.Entry(controls, width=16)

a_entry.insert(0, "25.75")

a_entry.grid(row=0, column=1)

ttk.Label(controls, text="Operation").grid(row=0, column=2, padx=5)

op_var = tk.StringVar(value="+")

ttk.Combobox(controls, textvariable=op_var, values=["+", "-"],

state="readonly", width=6).grid(row=0, column=3)

ttk.Label(controls, text="Sensor Reading B").grid(row=0, column=4, padx=5)

b_entry = ttk.Entry(controls, width=16)

b_entry.insert(0, "12.5")

b_entry.grid(row=0, column=5)

tree = self.make_tree(page,

("item", "decimal", "sign", "exponent", "exp_val", "mantissa", "bits"),

("Value", "Decimal", "Sign", "Exponent Bits", "Exp", "Mantissa", "32-bit IEEE"),

height=7)

explanation = tk.Text(page, height=13, wrap="word")

explanation.pack(fill="both", expand=True, padx=10, pady=8)

def run():

try:

a = float(a_entry.get())

b = float(b_entry.get())

result = a + b if op_var.get() == "+" else a - b

clear_tree(tree)

for name, value in [("A", a), ("B", b), ("Result", result)]:

bits, sign, exp_bits, exp, mantissa = ieee_fields(value)

tree.insert("", "end", values=(

name, value, sign, exp_bits, exp, mantissa, bits

))

_, _, exp_a_bits, exp_a, _ = ieee_fields(a)

_, _, exp_b_bits, exp_b, _ = ieee_fields(b)

shift = abs(exp_a - exp_b)

if math.isnan(result):

edge = "Special value: NaN"

elif math.isinf(result):

edge = "Edge case: Overflow to Infinity"

elif result != 0.0 and abs(result) < 1.17549435e-38:

edge = "Edge case: Subnormal / Underflow region"

else:

edge = "Normal IEEE-754 value"

explanation.delete("1.0", "end")

explanation.insert("end",

"IEEE-754 SIMULATION STEPS\n\n"

f"1. Convert A = {a} and B = {b} into single-precision IEEE-754 format.\n"

f"2. Compare exponents: A exponent = {exp_a}, B exponent = {exp_b}.\n"

f"3. Exponent alignment requires a mantissa shift of approximately {shift} bit position(s).\n"

f"4. Perform mantissa {('addition' if op_var.get() == '+' else 'subtraction')} with the aligned exponents.\n"

f"5. Normalize the resulting significand and round it to single precision.\n"

f"6. Final decimal result = {result}.\n\n"

f"Validation / Edge Case: {edge}\n\n"

"Note: Python struct is used to display the actual IEEE-754 single-precision bit pattern."

)

except OverflowError:

messagebox.showerror("Overflow", "The entered value cannot be represented as Python float.")

except Exception as e:

messagebox.showerror("Input Error", str(e))

ttk.Button(controls, text="Simulate IEEE-754", command=run).grid(row=0, column=6, padx=12)

# ----------------------------- PIPELINE ----------------------------------

def show_pipeline(self):

page = self.new_page()

ttk.Label(page, text="5-Stage Pipeline Hazard Simulator",

style="Title.TLabel").pack(pady=12)

top = ttk.Frame(page)

top.pack(fill="x", padx=10)

left = ttk.LabelFrame(top, text="Instruction Sequence (one instruction per line)", padding=8)

left.pack(side="left", fill="both", expand=True, padx=(0, 5))

instructions_text = tk.Text(left, height=9, width=55)

instructions_text.pack(fill="both", expand=True)

instructions_text.insert("1.0",

"LOAD R1, SENSOR1\n"

"LOAD R2, SENSOR2\n"

"ADD R3, R1, R2\n"

"MUL R4, R3, R5\n"

"STORE R4, MEMORY"

)

right = ttk.LabelFrame(top, text="Mitigation", padding=12)

right.pack(side="right", fill="y", padx=(5, 0))

forwarding_var = tk.BooleanVar(value=True)

ttk.Checkbutton(right, text="Enable Forwarding",

variable=forwarding_var).pack(anchor="w", pady=5)

ttk.Label(right,

text="Forwarding reduces many RAW data hazards.\n"

"A load-use dependency may still need one stall.",

wraplength=240).pack(pady=8)

summary_var = tk.StringVar(value="")

ttk.Label(right, textvariable=summary_var, style="Result.TLabel",

wraplength=240).pack(pady=8)

pipe_tree = self.make_tree(page, ("instruction",), ("Instruction",), height=9)

hazard_tree = self.make_tree(page, ("consumer", "producer", "type", "description"),

("Consumer #", "Producer #", "Hazard", "Description"), height=6)

def run():

try:

lines = instructions_text.get("1.0", "end").strip().splitlines()

rows, hazards, cycles, stalls = simulate_pipeline(lines, forwarding_var.get())

matrix, total = build_pipeline_matrix(rows)

columns = ["instruction"] + [f"c{i}" for i in range(1, total + 1)]

pipe_tree.destroy()

nonlocal_pipe[0] = self.make_tree(

page, columns,

["Instruction"] + [f"C{i}" for i in range(1, total + 1)],

height=max(5, len(rows) + 2)

)

tree = nonlocal_pipe[0]

for r, values in zip(rows, matrix):

tree.insert("", "end", values=[r["instruction"]] + values)

clear_tree(hazard_tree)

for h in hazards:

hazard_tree.insert("", "end", values=h)

mode = "Forwarding ENABLED" if forwarding_var.get() else "Forwarding DISABLED"

summary_var.set(f"{mode}\nTotal cycles: {cycles}\nStalls inserted: {stalls}")

except Exception as e:

messagebox.showerror("Pipeline Error", str(e))

nonlocal_pipe = [pipe_tree]

ttk.Button(right, text="Run Pipeline", command=run).pack(pady=10)

# ------------------------------- CACHE -----------------------------------

def show_cache(self):

page = self.new_page()

ttk.Label(page, text="Cache Memory and AMAT Simulator",

style="Title.TLabel").pack(pady=12)

controls = ttk.LabelFrame(page, text="Cache Configuration", padding=10)

controls.pack(fill="x", padx=10)

ttk.Label(controls, text="Memory Access Pattern").grid(row=0, column=0, padx=5, pady=5)

access_entry = ttk.Entry(controls, width=45)

access_entry.insert(0, "0, 4, 8, 0, 16, 4, 20, 0, 24, 8, 0")

access_entry.grid(row=0, column=1, columnspan=4, padx=5)

ttk.Label(controls, text="Cache Type").grid(row=1, column=0, padx=5)

type_var = tk.StringVar(value="Direct-Mapped")

ttk.Combobox(controls, textvariable=type_var,

values=["Direct-Mapped", "2-Way Set-Associative"],

state="readonly", width=24).grid(row=1, column=1)

ttk.Label(controls, text="Lines / Sets").grid(row=1, column=2, padx=5)

size_entry = ttk.Entry(controls, width=10)

size_entry.insert(0, "4")

size_entry.grid(row=1, column=3)

ttk.Label(controls, text="Block Size").grid(row=1, column=4, padx=5)

block_entry = ttk.Entry(controls, width=10)

block_entry.insert(0, "1")

block_entry.grid(row=1, column=5)

ttk.Label(controls, text="Hit Time (ns)").grid(row=2, column=0, padx=5)

hit_entry = ttk.Entry(controls, width=10)

hit_entry.insert(0, "1")

hit_entry.grid(row=2, column=1)

ttk.Label(controls, text="Miss Penalty (ns)").grid(row=2, column=2, padx=5)

penalty_entry = ttk.Entry(controls, width=10)

penalty_entry.insert(0, "10")

penalty_entry.grid(row=2, column=3)

summary_var = tk.StringVar(value="Run the simulation to calculate Hit Ratio, Miss Ratio and AMAT.")

ttk.Label(page, textvariable=summary_var, style="Result.TLabel",

wraplength=1000).pack(pady=8)

tree = self.make_tree(page,

("address", "block", "set", "tag", "result"),

("Address", "Block", "Cache Set", "Tag", "Result"), height=12)

comparison = ttk.Label(page, text="", justify="left", wraplength=1000)

comparison.pack(padx=15, pady=5, anchor="w")

def run():

try:

accesses = parse_accesses(access_entry.get())

size = int(size_entry.get())

block_size = int(block_entry.get())

hit_time = float(hit_entry.get())

miss_penalty = float(penalty_entry.get())

if size <= 0 or block_size <= 0:

raise ValueError("Lines/Sets and block size must be positive.")

if type_var.get() == "Direct-Mapped":

results = direct_mapped(accesses, size, block_size)

alternate = two_way_lru(accesses, max(1, size // 2), block_size)

current_name = f"Direct-Mapped ({size} lines)"

alternate_name = f"2-Way Set-Associative ({max(1, size // 2)} sets)"

else:

results = two_way_lru(accesses, size, block_size)

alternate = direct_mapped(accesses, size * 2, block_size)

current_name = f"2-Way Set-Associative ({size} sets)"

alternate_name = f"Direct-Mapped ({size * 2} lines)"

clear_tree(tree)

for row in results:

tree.insert("", "end", values=row)

hits, misses, hit_ratio, amat = cache_metrics(results, hit_time, miss_penalty)

ah, am, ar, aamat = cache_metrics(alternate, hit_time, miss_penalty)

summary_var.set(

f"{current_name}: Hits = {hits}, Misses = {misses}, "

f"Hit Ratio = {hit_ratio*100:.2f}%, Miss Ratio = {(1-hit_ratio)*100:.2f}%, "

f"AMAT = {amat:.2f} ns"

)

comparison.config(

text=(

f"Comparison Configuration: {alternate_name}\n"

f"Alternate Hit Ratio = {ar*100:.2f}% | Alternate AMAT = {aamat:.2f} ns\n"

f"Trade-off: The configuration with higher hit ratio generally reduces AMAT for the same "

f"hit time and miss penalty."

)

)

except Exception as e:

messagebox.showerror("Cache Error", str(e))

ttk.Button(controls, text="Run Cache Simulation", command=run).grid(

row=2, column=5, padx=12)

# ------------------------------ I/O --------------------------------------

def show_io(self):

page = self.new_page()

ttk.Label(page, text="I/O Interfacing Strategy Evaluator",

style="Title.TLabel").pack(pady=12)

config = ttk.LabelFrame(page, text="Sensor Streaming Configuration", padding=15)

config.pack(fill="x", padx=20)

ttk.Label(config, text="Sample Rate (samples/sec)").grid(row=0, column=0, padx=5, pady=5)

rate_entry = ttk.Entry(config, width=15)

rate_entry.insert(0, "1000")

rate_entry.grid(row=0, column=1)

ttk.Label(config, text="Bytes per Sample").grid(row=0, column=2, padx=5)

bytes_entry = ttk.Entry(config, width=15)

bytes_entry.insert(0, "4")

bytes_entry.grid(row=0, column=3)

ttk.Label(config, text="CPU Interrupt Cost (µs)").grid(row=1, column=0, padx=5)

int_cost_entry = ttk.Entry(config, width=15)

int_cost_entry.insert(0, "8")

int_cost_entry.grid(row=1, column=1)

ttk.Label(config, text="DMA Setup Cost (µs/block)").grid(row=1, column=2, padx=5)

dma_cost_entry = ttk.Entry(config, width=15)

dma_cost_entry.insert(0, "20")

dma_cost_entry.grid(row=1, column=3)

tree = self.make_tree(page,

("strategy", "cpu_time", "interrupts", "latency", "power"),

("Strategy", "CPU Service / sec", "Events / sec",

"Typical Latency", "Power View"), height=5)

recommendation = tk.Text(page, height=13, wrap="word")

recommendation.pack(fill="both", expand=True, padx=20, pady=10)

def run():

try:

rate = float(rate_entry.get())

bytes_per_sample = float(bytes_entry.get())

int_cost = float(int_cost_entry.get())

dma_cost = float(dma_cost_entry.get())

if rate <= 0 or bytes_per_sample <= 0:

raise ValueError("Sample rate and bytes per sample must be positive.")

interrupt_cpu_us = rate * int_cost

block_size = 32

dma_blocks = math.ceil(rate / block_size)

dma_cpu_us = dma_blocks * dma_cost

clear_tree(tree)

tree.insert("", "end", values=(

"Interrupt-Driven",

f"{interrupt_cpu_us:.2f} µs/sec",

f"{rate:.0f} interrupts/sec",

"Low per-sample response delay",

"Higher CPU wake-ups"

))

tree.insert("", "end", values=(

"DMA-Based",

f"{dma_cpu_us:.2f} µs/sec",

f"{dma_blocks:.0f} block events/sec",

"Block/buffer dependent",

"Lower CPU activity for streams"

))

if rate >= 500 or interrupt_cpu_us > dma_cpu_us * 1.5:

choice = "DMA-Based I/O"

reason = (

"The continuous sample stream makes repeated interrupts expensive. "

"DMA can move blocks of ADC samples into memory while reducing CPU wake-ups."

)

else:

choice = "Interrupt-Driven I/O"

reason = (

"For a low-rate or irregular sensor stream, interrupt-driven I/O offers simpler control "

"and low response delay without requiring block-buffer management."

)

throughput = rate * bytes_per_sample

recommendation.delete("1.0", "end")

recommendation.insert("end",

"I/O STRATEGY ANALYSIS\n\n"

f"Data throughput = {throughput:.2f} bytes/sec.\n"

f"Interrupt CPU service time = {interrupt_cpu_us:.2f} µs/sec.\n"

f"DMA CPU setup time = {dma_cpu_us:.2f} µs/sec (assuming {block_size}-sample blocks).\n\n"

f"Recommended Strategy: {choice}\n\n"

f"Justification: {reason}\n\n"

"For a battery-powered IoT edge node, reducing unnecessary CPU wake-ups is generally "

"important for energy efficiency, while latency requirements determine whether small "

"interrupt-driven transfers or buffered DMA transfers are more appropriate."

)

except Exception as e:

messagebox.showerror("I/O Error", str(e))

ttk.Button(config, text="Evaluate I/O Strategy", command=run).grid(

row=2, column=0, columnspan=4, pady=10)

if __name__ == "__main__":

app = NanoSenseApp()

app.mainloop()

OUTPUT :